Fluidigm Access Array Ribosomal Amplicon Assays

Requirements for submission

Extraction protocols – We recommend MO BIO bacterial or soil DNA extraction kits. Other protocols are also acceptable.

DNA Quality- We request submission of a gel picture showing high molecular weight Genomic DNA.

Amount to submit- Ideally, at least 15 ul of 20-40 ng/ul if measuring with nanodrop. (Please note that nanodrop measurements below 20 ng/ul are extremely inaccurate).  We will use 2 ul for Qubit high sensitivity concentration measures and dilute to working concentration.

260/230 and 260/280 ratios should optimally be between 1.7-2.0. Ratios diverging from this range may contain contaminants or proteins that can affect the efficiency of PCR amplification.

Samples must be supplied in well-sealed 96 well PCR plates with 48 samples in the first 6 columns. Please use any cone shaped well, no flat bottom wells.

An excel spreadsheet with the plate layout and sample IDs is required.

A unique ID is required for each well. Please label replicate samples with A or B extentions.

Shipping- Samples should be shipped overnight on dry ice to the following address:

Functional Genomics Laboratory

University of Illinois

1201 W. Gregory Dr.

ERML 356

Urbana, IL 61801

Tel: 217 244 3930  Attn: Mark Band

Please provide shipper and tracking number to markband@illinois.edu or akraiko@illinois.edu .

Please package plates in appropriate packing material to avoid damage or leakage.

Number of samples

Access Arrays are run with up to 48 samples each. No discounts will be credited for fewer than 48 samples.

Primer choices- Target Specific Primer 16s or 18S regions are listed below. (We also have a number of targets for soil samples that may be of interest.  Custom primers can be designed and the researcher will be charged an additional $100 for synthesis, shipping, and testing for the first 3 primer sets and $50 for each additional set, prior to Access Array run. We currently can run Fluidigm Access Arrays at annealing temperature of 60C or 55C so Tm of custom primers should be targeted appropriately. Note that all primers are run simultaneously with the same PCR cycling program.

Default Primer Options   (all primers are run by default at annealing 550 C)

Please note that the bioanalyser sizes are about 104 bases longer than the actual sequenced amplicon, due to the Fluidigm and Illumina linker extensions.

Fluidigm CS1 Adapter

Fluidigm CS2 Adapter

Underlined = Target Primer Region

Please note the revision to the V4 primers. By default we will be using these revised primers unless client specifically requests the old set.

V1-V3  643 bp Bioanalyzer

CS1-V1-V3 F28-2-for	ACACTGACGACATGGTTCTACAGAGTTTGATCNTGGCTCAG

CS2-V1-V3 R519-2-rev	TACGGTAGCAGAGACTTGGTCTGTNTTACNGCGGCKGCTG

V3-V5  694 bp  Bioanalyzer

CS1-V3-V5 F357-1-for	ACACTGACGACATGGTTCTACACCTACGGGAGGCAGCAG

CS2-V3-V5 R926-1-rev	TACGGTAGCAGAGACTTGGTCTCCGTCAATTCMTTTRAGT

V3-V4 ~540 bp Bioanalyzer

V3-357F      ACACTGACGACATGGTTCTACACCTACGGGNGGCWGCAG

V4-805R     TACGGTAGCAGAGACTTGGTCTGACTACHVGGGTATCTAATCC

V4     ~400 bp Bioanalyzer  (Old set)

V4-515F	ACACTGACGACATGGTTCTACAGTGCCAGCMGCCGCGGTAA

V4-806R	TACGGTAGCAGAGACTTGGTCTGGACTACHVGGGTWTCTAAT

V4   (revised)  ~400 bp Bioanalyzer  (The earth microbiome consortium has recently recommended these changes

V4-515F	ACACTGACGACATGGTTCTACAGTGYCAGCMGCCGCGGTAA

V4-806R	TACGGTAGCAGAGACTTGGTCTGGACTACNVGGGTWTCTAAT

Archaea 528 bp Bioanalyzer

CS1-Arch349F-3-for	ACACTGACGACATGGTTCTACAGYGCASCAGKCGMGAAW

CS2-Arch806R-3-rev	TACGGTAGCAGAGACTTGGTCTGGACTACVSGGGTATCTAAT

Euk566	765 + bp Bioanalyzer

CS1-F566Euk-4-for	ACACTGACGACATGGTTCTACACAGCAGCCGCGGTAATTCC

CS2-R1200Euk-4-rev	TACGGTAGCAGAGACTTGGTCTCCCGTGTTGAGTCAAATTAAGC

ITS1-ITS4  580  + bp Bioanalyzer

CS1-ITS1-10-for	ACACTGACGACATGGTTCTACATCCGTAGGTGAACCTGCGG

CS2-ITS4-10-rev	TACGGTAGCAGAGACTTGGTCTTCCTCCGCTTATTGATATGC

ITS3-ITS4 462 + bp Bioanalyzer

CS2-ITS3F-8-for 	ACACTGACGACATGGTTCTACAGCATCGATGAAGAACGCAGC

CS2-ITS4R-10-rev	TACGGTAGCAGAGACTTGGTCTTCCTCCGCTTATTGATATGC

18S 200-279 bp Bioanalyzer

CS1-Euk_1391f-7-for	ACACTGACGACATGGTTCTACAGTACACACCGCCCGTC

CS2-EukBr-7-rev	TACGGTAGCAGAGACTTGGTCTTGATCCTTCTGCAGGTTCACCTAC

Below are Bioanalyzer profiles for a number of primer pairs listed above. Fragment sizes are approximately 105 bp longer than the targeted amplicon due to the Fluidigm CS1 and CS2 tails as well as the Illumina barcodes and linkers.  Note that certain metacommunities may show varying amplicon lengths.